T7 series test instructions

Content

1.Wiring instruction	1

2.Wi-Fi fixed frequency test	2

3.BLE fixed frequency test	7

4.Wi-Fi adaptive test	10

5.BLE RX Blocking test	14

- 1.Wiring instruction

1.1 Module welding position

- T7-2S                  T7-U、T7-U-IPEX

1.2 Module  --Connect -- UART Board(TX\RX is reversed)

3V3  ------------------- 3.3V

GND  ------------------- GND

RX0  ---------------------TX

TX0  -------------------- RX

2.Wi-Fi fixed frequency test

- 2.1 UART driver：

- (Double click to download)

2.2 Use the Test Tool v1.9.2

(Double click to download)

- 2.3 Open the tool , select COM, Baud Rate: 115200, click Connect Port, IDLE becomes OK

- 2.4 Next, you can change Band, Standard, Channel,Data Rate,Bandwidth, Select WiFi-Tx, FCC/SRRC select CE/FCC/SRRC, or specifies specific values（eg.99）to adjust the duty cycle. Temp Cali : FALSE.

- (Do not click SAVE IN FLASH throughout the test.)

- 2.5 Select through the above settings and press Start to start the module to work. You can see the power index value, transmission rate, channel and other information through the log in the lower right frame, which means that the module is already in working state.

- 2.6 If you need to adjust the power, change the TX Pwr value to adjust the power. If the test value does not change after modification, click stop and then click start to test.

2.7 Wi-Fi RU Test

Use TUYA's instruments to illustrate the test method.During the actual certification,Test with the instrument of the testing institution

1）Set Standard to 11AX and PPDU Type to HE-TB, as shown in the figure：

2）First, connect itest and open itest test software, as shown in the figure：

Upper-right 设备设置->”分析设置”，Follow the Settings in the image below：

RU Index corresponds to RU Allocation in V1.7.8.1, and MCS corresponds to Data Rate in V1.7.8.1. This parameter must be consistent with the software Settings；

Such as, the Data Rate Settings are the same: MCS7;The RU Allocation Settings are the same: 0

3）By clicking start, itest can parse Wi-Fi 11AX's signal normally. If the test fails, you can manually set TXPwr, the range is 0~20 (it is determined according to the absolute power, the maximum transmission power of the chip is 20dbm, so the maximum tool can only be set to 20, more than 20 is invalid)

- 2.7 Wi-Fi Single Tone:TX setting CW selects TRUE first, then Testing Item selects WiFi-TX, click Start.

If the test fails, adjust the Xtal C value to test. If the waveform does not change after adjustment, click Stop and then start.

2.8 Wi-Fi RX

- 3.BLE fixed frequency test

- 3.1 The fixed frequency module is connected to the PC through the UART. Open the test tool, connect the port.

- 3.2 Select Bluetooth-TX,BLE patter select Continuous PRBS9 or Burst PRBS9, change Channel, TXPwr Default Auto, click Start；If you need to adjust the power, change the TxPwr value；

- 3.3 BLE Frequency error:Testing Item select Bluetooth-Tx,BLE pattern select Continuous wave, change Channel, click Start to test.

- 3.4 BLE RX

- Or can run commands to test BLE RX

- The fixed frequency module connects to the UART, insert the PC, Open the SSCOM

Double click to download

- To use the SSCOM tool, perform the following steps: Click EXT to appear the command

- Click "进入BLE DUT" to send the command：ble dut. As shown below

- Click “BLE RX 2402M” to send CH0; Send channel according to the actual test

- Then use the signal source to send 1M mode, single transmission, 1000 packets at a time

- Finally click “BLE stop”

- Click HEXshow , The return value is displayed in hex

- If after clicking Ble stop, the last four digits of the returned hexadecimal number are 0A 02, then the actual number of packets received is calculated as 0x02*256+0x0A

- 4.Wi-Fi adaptive test

- 4.1 The fixed frequency module is connected to the PC through the UART. Open the test tool, connect the port.

4.2 Send sta ssid psk to connect to the router

sta ssid psk      //ssid：router name   psk :router password

- Double click to download tool

4.3 Open the SSCOM tool and follow steps 1-6: Click EXT to appear the command

4.4 Click "7无注释" to send the command：sta CMW-AP 12345678

Connection succeed；Module IP: 192.168.178.173

- 4.5 The PC (-c) connects to the same network. After the connection is successful, view the IP address through network properties. PC IP：192.168.178.64

4.6 Download the iperf.zip

- （Double-click to download it to your desktop）

- 4.7 Run cmd in administrator mode；

- 4.8 Find the following file in the iperf.Zip downloaded in Step 4.7；

- 4.9 Click the down arrow to copy and paste the path; Then add cd in front of the path；

- 4.10 Send instruction: iperf -s -i 1；

- 4.11 Module: SSCOM send instruction: iperf -c 192.168.178.64.

- The ip address is changed based on the actual PC address；

- The default is 30 packages

- iperf -c 192.168.178.64 -t 10 -i 1  (You can change 10 to the number of packets you want)

- 4.12 Module as receive，SSCOM：iperf -s, cmd （PC）：iperf -c Module IP -t 10 -i 1

- Module IP：192.168.178.173, so：iperf -c 192.168.178.173 -t 10 -i 1；

The module IP address is subject to the actual test module IP address

5.BLE RX Blocking test

- 5.1 The module VCC and GND are connected to CMW500, modules TX0, RX0 and GND are connected to the UART and inserted to the PC to send command.

5.2 Click EXT to appear the command, click "进入BLE DUT" to send the command：ble dut

- 5.3 TX0 and RX0 are connected to the serial port of the CMW，Don't cut off the power halfway

- 5.4 Operate CMW500 Signaling test RX Blocking，setting as below：