T3AI-V test instructions

Content

1.Wiring instruction	1

2.Wi-Fi fixed frequency test	2

3.BLE fixed frequency test	8

4.Wi-Fi adaptive test	12

5.BLE RX Blocking test (Signaling)	16

- 1.Wiring instruction

1.1 Module welding position

1.2 Fixed frequency module  --Connect -- UART Board(TX\RX is reversed)

VBAT  -------------------------------------  3.3V

GND  ------------------------------------  GND

RX0  ---------------------------------------  TX

TX0  ---------------------------------------- GND

After the UART board is plugged into the PC, Module TX0 connects to UART RX

2.Wi-Fi fixed frequency test

- 2.1 After connecting the fixed frequency module as described above, download the UART driver

- UART driver：(The computer recognizes COM, you can not install the driver)

- (Double click to download)

2.2 Use the Wifi Test Tool v1.9.0

(Double click to download)

- 2.3 Open the tool , select COM, Baud Rate: 115200, click Connect Port, IDLE becomes OK

- 2.4 Testing Item selected: WiFi-Tx, you can change Standard, Bandwidth, Data Rate, Channel,  FCC/SRRC need to select FCC, CE or SRRC, and It can also input specific values to adjust the duty cycle. Temp Cali : FALSE. Then click on "Start" to begin the test..

- (Do not click SAVE IN FLASH throughout the test.)

- 2.5 If you need to adjust the power, change the TX Pwr value to adjust the power. If the test value does not change after modification, click stop and then click start to test.

- Adjust the TXPwr: log

- 2.6 Wi-Fi Single Tone Test: TX setting CW selects TRUE, then Testing Item selects WiFi-TX, click Start to test.

If the test fails, adjust the Xtal C value to test.

For Example: select 10, Log：

2.7 Wi-Fi RU Test

Use TUYA's instruments to illustrate the test method.During the actual certification,Test with the instrument of the testing institution

1）Set Standard to 11AX and PPDU Type to HE-TB, as shown in the figure：

2）First, connect itest and open itest test software, as shown in the figure：

Upper-right 设备设置->”分析设置”，Follow the Settings in the image below：

RU Index corresponds to RU Allocation in V1.7.8.1, and MCS corresponds to Data Rate in V1.7.8.1. This parameter must be consistent with the software Settings；

Such as, the Data Rate Settings are the same: MCS7;The RU Allocation Settings are the same: 0

3）By clicking start, itest can parse Wi-Fi 11AX's signal normally. If the test fails, you can manually set TXPwr, the range is 0~20 (it is determined according to the absolute power, the maximum transmission power of the chip is 20dbm, so the maximum tool can only be set to 20, more than 20 is invalid)

2.8 Wi-Fi RX test

- 3.BLE fixed frequency test

- 3.1 The module is connected to the UART according to the wiring instructions. Open the tool, connect the port.

- 3.2 Testing Item selected: Bluetooth-TX,BLE patter select Continuous PRBS9 or Burst PRBS9, change Channel, Rate(support LE1M, S8, S2), TXPwr default Auto, click Start；If you need to adjust the power, change the TxPwr value；

- 3.3 If there is a problem with the test waveform, download SSCOM and perform the BLE TX test via the command method.

- (Double click to download)

- Select the Com, set the baud rate to 115200, open the Com, and click "EXT" to display the written command.

- Click "Enter BLE DUT" to send the command "ble dut". As shown in the following figure:

- Click HEXshow

- Then send the TX test command

- After completing one test, you need to input the Stop command before proceeding to the next test：01 1F 20 00

- Adjust the power：01 E0 FC 02 10 01  (Range 0-7F)

- 3.4 BLE Frequency error:Testing Item select Bluetooth-Tx,BLE pattern select Continuous wave, change Channel, click Start to test.

- 3.5 BLE RX: select Bluetooth-Rx, click Start to test.

- 3.6 Or can run commands to test BLE RX，Use the SSCOM

If it has already been downloaded, there is no need to download it again.

- Open the SSCOM tool, Select the Com, set the baud rate to 115200, open the Com, and click "EXT" to display the written command.

- Click "进入BLE DUT" to send the command：ble dut

- Click HEXshow, click “BLE RX 2402M” to send CH0; Send channel according to the actual test

- Then use the signal source to send 1M mode, single transmission, 1000 packets at a time

- Finally click “BLE stop”

- As shown above,clicking Ble Stop, the last four digits of the returned hexadecimal number are 0A 02, then the actual number of packets received is calculated as 0x02*256+0x0A,it is 522 packets.

- Test CH0 in the following figure, the last four digits are 7D 03, and the number of packets received is 893

- 4.Wi-Fi adaptive test

4.1 The module is connected to the UART according to the wiring instructions.

4.2 Use the SSCOM tool

- Double click to download tool

4.3 Open the SSCOM tool and follow steps 1-3: Click EXT to appear the command

4.4 Send command (sta ssid psk) to connect to the router

sta ssid psk      //ssid：router name   psk :router password

For example：sta CMW-AP 12345678

4.5 Click "5无注释" to send the command: sta CMW-AP 12345678

Connection succeed; send command to view module IP: ipconfig

As shown in the figure Module IP: 192.168.127.233

- 4.6 The PC (-c) connects to the same network. After the connection is successful, view the IP address through network properties. PC IP：192.168.178.64

4.7 Download the iperf.zip

- Double-click to download it to your desktop

- 4.8 Run cmd in administrator mode；

- 4.9 Find the following file in the iperf.Zip downloaded in Step 4.7；

- 4.10 Click the down arrow to copy and paste the path; Then add cd in front of the path；

- 4.11 Send instruction: iperf -s -i 1；

- 4.12 Module: SSCOM send command: iperf -c 192.168.178.64

- The ip address is changed based on the actual PC address；

- The default is always running. Stop sending command: iperf --stop

- 4.13 Module as receive, SSCOM: iperf -s, cmd（PC）：iperf -c Module IP

- Module IP：192.168.178.233, so：iperf -c 192.168.178.233

The module IP address is subject to the actual test module IP address

The default is 10 packages

PC：iperf -c 192.168.127.233 -t 20 -i 1

20 can be changed to the number of packets required

5.BLE RX Blocking test (Signaling)

5.1 Wiring as follows

Module ----- UART1 connect to the PC USB

GND  ----  GND

RX0  ----  TX

TX0  ----  GND

- Module ----- UART2 connect to the CMW500 USB

3V3  ----  3.3V    (T3-V-PRO VBAT Connect UART 5V)

GND  ----  GND

- After UART1 is connected to the computer and UART2 is connected to the CMW500, TX0 of UART1 is connected to the UART1 RX, and the power is kept on during the process

5.2 Click EXT to appear the command, click "进入BLE DUT" to send the command：ble dut

- 5.3 TX0 and RX0 are connected to the USB of the CMW, Don't cut off the power halfway

- 5.4 Operate CMW500 Signaling test RX Blocking，setting as below：